Smart Spoilage Detection and Prevention System for Perishable Goods Using IoT Sensors and Machine Learning

Rithuparan PS 1, Harshavardhan.R2  ,Dr.Geetha.C* 3

Dept of Computer Science and Engineering, R.M.K. Engineering College, Chennai, India.

Corresponding Author: Dr.Geetha C(cga.cse@rmkec.ac.in)

Abstract:

Food spoilage in perishable goods storage poses a significant challenge, leading to massive financial losses, food wastage, and supply chain inefficiencies. Current monitoring approaches—including manual inspections and basic sensor-based systems—lack the real-time, predictive insights necessary for proactive intervention. This paper proposes an IoT-integrated Smart Spoilage Detection and Prevention System that combines a multi-sensor grid, real-time data acquisition, and machine learning (ML)-based predictive analytics. The system deploys temperature (DHT22), humidity, volatile organic compounds (VOC) (MQ-135), and CO₂ sensors (MH-Z19B) across storage racks to capture environmental data continuously. An ESP32 microcontroller processes the sensor data and employs an Isolation Forest algorithm for unsupervised anomaly detection, thereby predicting spoilage trends before visible deterioration occurs. Integrated with a cloud-based dashboard and mobile application, the system issues automated inventory management suggestions and real-time alerts. Experimental evaluations demonstrate a spoilage detection accuracy of 94.2%, a reduction in food wastage by 30%, and a response latency of under 100ms. This work establishes a scalable, cost-efficient, and highly accurate framework for food storage monitoring and spoilage prevention.

Keywords:

IoT, Machine Learning, Spoilage Detection, Food Preservation, Perishable Goods Monitoring, Real-Time Alerts

# INTRODUCTION

The integrity of the global food supply chain is significantly compromised by product deterioration. According to the Food and Agriculture Organization (FAO), suboptimal storage infrastructure accounts for nearly 40% of worldwide food waste [1]. Ensuring the longevity of perishable goods—ranging from agricultural produce and dairy to pharmaceuticals—necessitates precise environmental regulation. Unfortunately, conventional surveillance strategies, which often rely on periodic physical checks or rudimentary sensing tools, are characterized by latency in identifying issues. Furthermore, these legacy systems fail to account for the intricate correlation between atmospheric variables, such as gas concentrations, humidity levels, and temperature, which collectively accelerate decay [2], [3].

Emerging capabilities in the Internet of Things (IoT) domain combined with Machine Learning (ML) algorithms present novel avenues for optimizing continuous surveillance and forecasting. The integration of distributed sensor arrays with predictive computational models facilitates the identification of incipient spoilage. This synergy allows for preemptive countermeasures to be taken before significant loss occurs. Consequently, the primary contributions of this study are to:

- Engineer a continuous monitoring architecture leveraging a decentralized sensor mesh to record environmental parameters including gas emissions, thermal conditions, and moisture levels.

- Deploy an algorithmic predictive framework (specifically Isolation Forest) that synthesizes both archival and live data streams to project future deterioration patterns.

- Construct a responsive notification mechanism linked to cloud-based visualization tools and mobile interfaces, thereby empowering warehouse administrators with actionable intelligence for rapid decision-making.

- Assess the proposed solution’s efficacy regarding the mitigation of food waste and the optimization of inventory control processes

# RELATED WORKS

## Traditional Spoilage Detection Methods

Conventional spoilage detection systems typically rely on manual inspection or basic environmental sensors (e.g., temperature-only monitors). Such systems are prone to high false- negative rates and do not address the spatial heterogeneity present in modern storage facilities [3].

## IoT-Based Monitoring Approaches

Several studies have integrated IoT sensors for food quality monitoring. Ishangulyey et al. [1] presented an IoT-based approach for spoilage detection in storage facilities, while Sonwani et al., [2] focused on edge computing solutions to process sensor data in real time. However, these systems generally lack advanced predictive analytics, which limits early spoilage detection capabilities [4].

## Machine Learning for Spoilage Prediction

ML techniques such as decision trees, neural networks, and anomaly detection algorithms have been applied to food quality monitoring. For instance, Ahmed et al. [4] demonstrated predictive analytics for spoilage prevention using cold storage data. However, many of these approaches require substantial computational resources, making them less viable for deployment on resource-constrained edge devices [5]. Our work builds on these studies by employing an optimized Isolation Forest algorithm that is both accurate and efficient on platforms like the ESP32 [6], [7].

# METHODOLOGY

## System Architecture

Fig. 1: Data flow diagram of the spoilage detection system. Environmental inputs from temperature, humidity, and VOC sensors are processed by a local unit to identify spoilage risks in real-time. The system integrates edge computing for immediate on-site alerts (graphs and alarms) with cloud connectivity for remote monitoring and mobile notification delivery.

The proposed system comprises four major components: a sensor grid network, a data processing unit, an ML-based predictive module, and an alert system with cloud integration.

A distributed sensor grid is deployed across storage shelves, consisting of:

    - DHT22 (Temperature & Humidity Sensor): Captures ambient temperature fluctuations and relative humidity levels.

    - MQ-135 (VOC Sensor): Measures volatile organic compounds such as ethylene, which are indicative of spoilage.

    - MH-Z19B (CO₂ Sensor): Monitors CO₂ levels, a marker of microbial activity and potential spoilage.

This multi-sensor fusion approach overcomes the limitations of single-point monitoring by capturing the spatial heterogeneity of storage environments [5], [8].

## Data Processing Unit

Sensor data is continuously transmitted to an ESP32 microcontroller that performs initial preprocessing. Key functions include:

    - Noise Filtering: Utilizing Kalman filtering to reduce sensor noise .

    - Threshold Comparison: Sensor readings are compared against predefined spoilage thresholds.

    - Data Transmission: Processed data is forwarded via Wi-Fi to the cloud for advanced analysis [9].

## Machine Learning-Based Spoilage Prediction

The ML module uses the Isolation Forest algorithm for anomaly detection. Given a dataset X={x1,x2,…,xn}, the anomaly score S(x) is computed as:

where E(h(x)) is the expected path length of point xxx in a randomly constructed tree, and c(n) is the average path length of an isolation tree in a balanced tree [10]. High values of S(x) indicate potential spoilage anomalies. The model integrates features such as:

    - Temperature-Humidity Correlations: To detect unsafe storage conditions.

    - VOC and CO₂ Patterns: To identify microbial spoilage risks.

    - Temporal Dynamics: Rate of change in environmental parameters.

## Alert System & Cloud Dashboard

The alert subsystem delivers notifications via:

    - Cloud-Based Dashboard: Provides real-time visualization of storage conditions and spoilage risk levels.

    - Mobile Notifications: Leveraging Firebase Cloud Messaging to alert warehouse managers immediately when anomalies are detected.

    - Automated Inventory Suggestions: Based on predictive analytics, the system advises on stock rotation and removal of at-risk goods.

# IMPLEMENTATION & EXPERIMENTAL SETUP

## Hardware Components

The system hardware includes:

    - ESP32 Microcontroller: For edge computing and data processing.

    - DHT22, MQ-135, and MH-Z19B Sensors: For environmental monitoring.

    - Wi-Fi Module: For data transmission to cloud services.

## Software Components

The software architecture comprises:

    - Python and TensorFlow: For implementing and training the ML model.

    - Flask API: To interface sensor data with cloud storage and the dashboard.

    - Firebase and MQTT: For real-time alerting and dashboard updates [11].

## Dataset & Model Training

Data was collected continuously over three months from a controlled storage environment. The dataset was supplemented with publicly available datasets on food spoilage conditions [12]. The Isolation Forest model was trained and fine-tuned to achieve optimal detection accuracy while maintaining low inference latency.

  - Experimental Setup

Experiments were conducted in three distinct storage environments:

    - Dairy Cold Storage: 2–4°C and 70% relative humidity.

    - Fruit Ripening Room: 12°C, 85% relative humidity, with elevated ethylene levels.

    - Vegetable Warehouse: 8°C and 75% relative humidity.

Key performance metrics such as detection accuracy, false positive rate, latency, and energy consumption were recorded [13].Results & Discussion

## Spoilage Detection Accuracy

The developed architecture attained a cumulative precision rate of 94.2% in identifying product deterioration. When contrasted with conventional limit-based monitoring (which holds an 82.5% fidelity), the machine learning strategy substantially minimized missed detection events (Type II errors) and enhanced the forecasting of incipient decay phases.

## Response Delay and Power Optimization

Performance metrics indicated an instantaneous reaction interval averaging 98ms (remaining consistently below 100ms), satisfying the stringent criteria necessary for preemptive waste mitigation. Power usage assessments demonstrated that our streamlined edge computation lowered energy expenditure by 42% relative to standard sensor nodes.

## Comparative Benchmarking

TABLE I
PERFORMANCE EVALUATION OF SPOILAGE DETECTION METHODOLOGIES

| Method | Accuracy | False Positives (%) | Latency (ms) |
| --- | --- | --- | --- |
| Proposed ML- Based System | 94.2% | 4.3% | 98ms |
| Threshold-Based Monitoring | 82.5% | 12.7% | 150ms |
| Manual Inspection | 65.1% | N/A | High |

The engineered solution surpassed legacy methodologies regarding both identification precision and operational speed, facilitating prompt corrective measures to curtail inventory loss.

## Analytical Review

Empirical testing validates that synthesizing data from a distributed sensor array with a refined Isolation Forest model offers reliable forecasting of deterioration incidents. The strengths of this methodology include:

- Sensor Data Synthesis: Aggregating varied atmospheric metrics enhances the resilience of the decay identification process [14].

- On-Device Computation: Lowering signal delay and power usage is vital for instantaneous functionality [15].

- Remote Connectivity: Enabling off-site supervision and automated warnings to optimize stock administration [16].

Notwithstanding these improvements, obstacles such as instrument calibration shifts and fluctuating external noise persist. Future studies will investigate self-adjusting calibration protocols and deep learning refinements to augment model stability [8], [17].

# CONCLUSION & PROSPECTIVE DEVELOPMENTS

This study details an extensive framework utilizing IoT and ML for the instantaneous identification of decay in cold storage environments. By utilizing a decentralized sensing mesh, a perimeter-optimized Isolation Forest algorithm, and a unified notification mechanism, the solution secures a 94.2% identification fidelity with sub-100ms delay, decreasing food loss by 30%.

Subsequent research initiatives will target:

- Collaborative Model Training (Federated Learning): To collectively refine algorithms across diverse warehouse locations while maintaining data confidentiality.

- Self-Regulating HVAC Integration: Linking predictive decay metrics with climate infrastructure for responsive atmospheric tuning.

- Next-Generation Instrumentation: Integrating supplementary modalities, such as hyperspectral scanning, for superior decay forecasting.

- Logistics Unification: Expanding the surveillance scope to transport segments and engineering blockchain-backed tracking for heightened quality control.

The progress outlined here offers a scalable, cost-effective structure connecting theoretical inquiry with practical industrial utility in inventory preservation [16], [18].

# REFERENCES

- Ishangulyey R, Kim A, LEE SH. 2019. Understanding Food Loss and Waste—Why Are We Losing and Wasting Food? Foods, 8(8):297. doi: 10.3390/foods8080297

- Sonwani E, Bansal U, Alroobaea R, Baqasah AM, Hedabou M. 2022. An Artificial Intelligence Approach Toward Food Spoilage Detection and Analysis. Front Public Health, Jan 12;9:816226

- Ma M, Yang X, Ying X, Shi C, Jia Z, Jia B. 2023. Applications of Gas Sensing in Food Quality Detection: A Review. Foods; 12(21): 3966.doi: 10.3390/foods12213966

- Dhal, S.B., Kar, D. 2025. Leveraging artificial intelligence and advanced food processing techniques for enhanced food safety, quality, and security: a comprehensive review. Discov Appl Sci 7, 75

- Qian C, Murphy ST, Orsi RH, Wiedmann M. 2023. How Can AI Help Improve Food Safety? Annual Review of Food Science and Technology, Vol 14, 517 – 538.

- Pandey VK, Srivastava S, Dash KK, Singh R, Mukarram SA, Kovács B, Harsányi E. 2023. Machine Learning Algorithms and Fundamentals as Emerging Safety Tools in Preservation of Fruits and Vegetables: A Review. Processes,11(6):1720.

- Hu, G.; Ahmed, M.; L’Abbé, M.R. 2023. Natural Language Processing and Machine Learning Approaches for Food Categorization and Nutrition Quality Prediction Compared with Traditional Methods. Am. J. Clin. Nutr., 117, 553–563.

- Hassan Anwar, Talha Anwar, Shamas Murtaza. 2023. Review on food quality assessment using machine learning and electronic nose system, Biosensors and Bioelectronics: X, Volume 14, 100365,

- Jwo D-J, Biswal A. 2023. Implementation and Performance Analysis of Kalman Filters with Consistency Validation. Mathematics.11(3):521.

- Priyanto, C.Y.; Purnomo, H.D. 2021. Combination of isolation forest and LSTM autoencoder for anomaly detection. In: proceedings of the 2021 2nd international conference on innovative and creative information technology (ICITech); IEEE: NY,USA.

- Manoj KS, Vignesh V, Gokul N. 2024. IOT Based Food Degradation Detection System. International Journal of Engineering Applied Sciences and Technology, 9 (8), ISSN No. 2455-2143,

- Karim AB, Alshehri AA, Tariq U, Salim F, Javed I. 2018. Monitoring food storage humidity and temperature data using IoT. MOJ Food Process Technology, 6:400–4.

- Yu B, Zhan P, Lei M, Zhou F, Wang P. 2020. Food quality monitoring system based on smart contracts and evaluation models. IEEE Access, 8:12479–90.

- Kumar A. 2020. An arduino sensor-based approach for detecting the food spoilage. Int J Eng Appl Sci Technol. 5:596–9.

- Nath S. 2024. Advancements in food quality monitoring: integrating biosensors for precision detection. Sustainable Food Technology.

- Malik PK, Sharma R, Singh R, Gehlot A, Satapathy SC, Alnumay WS, Pelusi D, Ghosh U and Nayak J. 2021. Industrial Internet of Things and its applications in industry 4.0: State of the art, Comput. Commun., 166, 125–139

- Song G, Zhang Z, Fauconnier M.-L, Li C, Chen L, Zheng X and Zhang D. 2023. Bimodal single-atom iron nanozyme biosensor for volatile amine and food freshness detection, Nano Today, 53, 102025

- Pal A and Kant K. 2020. Smart Sensing, Communication, and Control in Perishable Food Supply Chain. ACM Trans. Sen. Netw. 16, 1, Article 12